QUICK START GUIDE FOR webTRIM.FaTE VERSION 1.0.0, October 2026

This guide summarizes how users can access and use the current release of webTRIM.FaTE. For each of the interface pages referenced below, more detailed guidance on usage is available via on-screen help text.

Access the webTRIM.FaTE instance at: https://trim.epa.gov/

Click the “Sign in with Login.gov” button in the center of the landing page. After being redirected to Login.gov, sign in with your Login.gov credentials.

Click the New Scenario button. Enter your scenario name and optional description (you can edit them later if needed). Click Create.

Once you enter a modeling scenario, you will see a series of 14 tabs on the left side of the screen. We discuss each of these tabs in the remainder of this guide. These tabs represent the workflow of setting up a simulation. Each tab opens a page of information for you to interact with to set up your simulation. You should generally work through the tabs sequentially from first to last. You are always free to revisit tabs, but be aware that updating something on one tab may impact information on subsequent tabs. For example, going back to add another parcel to your simulation, or to change the nature of an existing parcel, will mean that you need to go into other tabs to populate or update information about that parcel as needed. As another example, changing the amount of rainfall in your scenario may impact the lake flush rates on the Water Bodies tab.

Default values are available for many of the model properties. In many cases, using default values will be sufficient for your simulation. Your parcels, meteorological data, leaf litterfall and “allow exchange” data, emissions data, and surface runoff data should always be based on site-specific information. We also recommend that several other soil and water body characteristics come from site-specific information where possible; those properties are specified in the text below.

General Info: On this tab, you can specify the duration of the simulation. For production runs, a 50-year period is recommended. For testing prior to a production run, a 10-year simulation period is typically sufficient. The User Access section (only visible to scenario managers) allows you to share access to your scenario with another user. You can also edit the scenario name and description from this tab, if desired.

Parcels: Use the map on this tab to define the parcels representing your scenario and define the land use of each parcel type in the table below.

- Features to have in mind when designing your parcels include:

  - Areas of higher chemical deposition from your emission sources (see below about importing and viewing AERMOD-generated deposition amounts)

  - Farms or hypothetical home garden areas

  - Fishable water bodies

  - Watershed boundaries

  - Areas of different land cover

  - Note: You can use the parcel map’s available base layers to identify candidates for the features described above and to help define the land use of each parcel.

- To reuse a set of parcels you made in another webTRIM.FaTE scenario, open the Parcels tab in the other scenario and export the parcel information to a geojson file by using the ↓ button on the left side of the parcel map. Then, in your new scenario, you can use either the Load Parcels from File form or the ↑ button on the map to import that file. The import may take a few minutes.

- The Deposition Overlay form allows you to import AERMOD .PLT file deposition results which will be displayed as an overlay on the map. This may assist in configuring parcels around higher deposition areas, which are the areas of most interest to most users. The .PLT file must contain either the total deposition amounts or the separate dry and wet deposition amounts. When importing, you must also indicate the UTM zone of the file’s receptor coordinates.

  - There is also a feature to configure, view, and download a receptor grid covering your parcels for use in an AERMOD run (this will be helpful on the Emissions page, as noted below).

- Use land cover information to identify and set the primary ground cover/vegetation type of each parcel. The National Land Cover Database (NLCD) is available as a base layer on the parcel map utility to assist with this.

Emissions: Use this tab to introduce chemical mass to the desired model compartments.

- Chemicals currently supported: mercury (elemental, divalent, and methyl are all added when you select “mercury”), cadmium, arsenic, 14 species of polycyclic organic matter (POM), and 17 dioxin/furan congeners.

  - Note that arsenic simulations do not model the aquatic food web. Instead, they use bioaccumulation factors on the Calculate Risk tab (see below) to estimate fish concentrations from webTRIM.FaTE’s modeled water and sediment concentrations.

- We strongly recommend that users run AERMOD on their emission sources of interest with receptors configured from the Parcels page (see above) to obtain AERMOD-generated air concentration, dry deposition, and wet deposition values. Running AERMOD is not facilitated within webTRIM.FaTE, so it must be run outside of webTRIM.FaTE. You can then import the AERMOD .PLT output files to webTRIM.FaTE by checking Use values from input file. The imported AERMOD data will be used to introduce the deposited chemical into model compartments. (These AERMOD air concentrations will be useful on the Calculate Risk tab, too; see below.)

  - When importing a .PLT file, be sure to input into webTRIM.FaTE the UTM zone of the receptor coordinates in your .PLT file.

  - You must import a .PLT file appropriate to the phase of the chemical you want to model, which is selectable on-screen (i.e., vapor-phase or particle-phase; if modeling both, import two .PLT files separately). For each parcel, the AERMOD deposition values will then be spatially joined into webTRIM.FaTE compartments called dryVaporSource and wetVaporSource (for vapor deposition) and dryParticleSource and wetParticlesource (for particle deposition). The values derived from the .PLT file will show up on-screen after import.

- Alternatively, you may introduce chemical deposition into webTRIM.FaTE by typing g/day deposition values into the desired model compartments, which include the four AERMOD-related compartments noted above as well as compartments like surface soil and surface water.

Meteorology: Use this tab to enter static (average) site-specific values for meteorological properties. Alternatively, you may import a site-specific time-series meteorological file (either as a CSV following the template provided on the Meteorology page as a download link, or as an AERMOD .SFC file) while specifying which column relates to which meteorological property.

- Note that webTRIM.FaTE will calculate and use averages from your imported time-series data. webTRIM.FaTE currently simulates with static (average) meteorological conditions.

Seasonal Dynamics: Use this tab to enter static (average) values for litterfall and leaf-air exchange (“allowExchange”) properties or to import time-series values by file.

- Note that webTRIM.FaTE will calculate and use averages from your imported time-series data. webTRIM.FaTE currently simulates with static (average) litterfall and allowExchange conditions.

Abiotic Media: Use this tab to specify the abiotic properties associated with the various air, soil, and groundwater compartments associated with each parcel.

- You can automatically import values for some abiotic properties within the Surface Soil Properties, Root Soil Properties, Vadose Zone Properties, and Groundwater Zone Properties subsections from online USDA resources, by clicking the Load from USDA Soil DataMart button. These values are: pH, organic carbon fraction, water content (not a property for the Groundwater Zone), and fraction sand. Auto-generation of values for these properties will occur where data are available.

- pH for surface soil is the one abiotic property highly recommended to receive a site-specific value (particularly if modeling metals), while the others may remain at their default values if time, resources, and data are limited. As noted above, the auto-generation feature will generate site-specific values for pH as well as the other properties listed above.

Surface Runoff: Use this tab to fill in the runoff matrix using site-specific hydrological information, if known.

- If these values are not known, the GetFlow feature (run by clicking the Run GetFlow button) will automatically calculate runoff between parcels based on local elevation data.

Water Bodies: Use this tab to enter site-specific property values, if known, for each water body.

- Water Column Properties recommended to receive site-specific values (if available) are water depth, flush rate, suspended sediment concentration, pH (particularly for modeling metals), algae density, organic carbon fraction in suspended sediments, and water temperature.

  - If you do not have a site-specific flush rate, click the on-screen Auto-calculate button to calculate a flush rate using other information you enter, such as precipitation on the Meteorology page and runoff values on the Surface Runoff page.

  - Algae density, if not directly available from site-specific sources, may be estimated from site-specific total phosphorus content if available:

    - (0.4*TPhos + 0.2*TPhos1.4) / WaterColumnDepth, where TPhos = average total phosphorus level (in µg/L or mg/m3; this is not a model input property), and WaterColumnDepth = the water column depth (in meters; this is a model input property).

- The Show/Hide Non-Interactive Data toggle switch on the top-right of the page can be used to reduce the information that is seen on the screen to only that which can be modified.

Aquatic Food Web: Use this tab to modify the aquatic biota biomass, body weights, and food web dynamics.

- Only standing biomass is recommended for a site-specific simulation. However, much of the information on this page (including biomass) is often not available for specific water bodies, and defaults may be the most reasonable option.

Erosion: Use this tab to specify the erosion dynamics of the scenario.

- If using the option to Supply Universal Soil Loss Equation (USLE) parameters (“Option 3” under Erosion data source), you have the option to auto-generate several factors from online USDA resources by clicking the Load USDA Soil Data button. The auto-generatable USLE factors are: erodibility index (K), slope gradient (S), slope length (L), and topographical length-slope factor (LS; calculated from S and L).

Background Concentrations: Use this tab to specify any pre-existing concentrations at the start of the simulation period.

- Note carefully the specific units which the application expects background concentration to be given in for each media type.

Summary of Properties: Use this tab to review your input properties for the simulation.

- Note: some model inputs are not currently viewable on this tab, such as properties calculated using model formulas, and compartment-specific chemical properties.

Run Transport Model: Use this page to initiate a chemical transport model run, download the results, and visualize the model output.

- A model run takes about 15 minutes to complete, depending on the domain size, number of parcels, and number of chemicals. There are progress trackers on-screen which will update periodically. When they all reach 100% and options are presented on-screen to visualize results, the model run has completed.

Calculate Risk: Use this page to estimate chemical dose, hazard quotient, and cancer risk based on the results of the fate-and-transport model along with additional user inputs (e.g., ingestion rates, biotransfer factors, and human-health toxicity values).

- Use the Edit Exposure Profiles button to view and edit existing profiles (sets of exposure and risk properties), or to develop a new profile.

  - A new profile can inherit properties from an existing profile.

- Use the Add Simulation button to build a risk simulation for a single chemical. In the pop-up window, select or enter the requested data. The selections you make and data you enter might be specific to:

  - the chemical (e.g., arsenic fish concentrations cannot be set directly, but instead must be calculated using bioaccumulation factors),

  - your method of providing emissions data to the transport model (i.e., via AERMOD output or not), or

  - the exposure scenario you are evaluating (e.g., which water body should be used for the fish consumption exposure pathway).

- Click Save when you have entered all required data.

- The risk calculations should be completed nearly instantaneously. You will then have the option to view the results using a built-in visualization and data table, or download them for external use.

- Repeat the above procedure for each applicable chemical and exposure scenario, as needed. There is a feature to clone an existing risk simulation by clicking the  button on the top right when viewing the simulation you want to clone. This will open a pop-up dialog identical to the one for creating a new simulation, pre-populated with values from the old simulation: you can keep or change any selections as needed (e.g., selecting a different chemical to calculate risk for).

General tips:

- Your inputs are saved as soon as they are entered. You can click the Recalculate button at the bottom of most tabs if the screen appears to hang or if you are unsure if your changes are showing up; your data will have been saved and will be visible. You can also refresh the page.

- You can use the default values for ease of testing.

- While waiting for the transport model to complete running, you can navigate to other pages or tabs and return to the Run Transport Model tab to view or download results later. Refreshing the page may help to reload the results if the page appears to be hanging.

- Expect run times for the transport model to range from ten minutes to one hour, depending on the complexity of the scenario.

https://www.epa.gov/scram/air-quality-dispersion-modeling-preferred-and-recommended-models